Chapter 1: Manufacturing Principles and Processes

- (Easy) What is the term for the optimum use of all resources including manpower, materials, machines, and energy?
a) Standardization
b) Productivity
c) Mechanization
d) Interchangeability

- (Easy) Which production method is most economical for high-volume runs of standard parts using specialized workforce?
a) Unit production
b) Batch production
c) Mass production
d) Job shop production

- (Normal) The concept of building quality into a product from the design stage onwards is central to:
a) Total Quality Management (TQM)
b) Just-in-Time (JIT)
c) Group Technology
d) Flexible Manufacturing System (FMS)

- (Difficult) According to the text, which of the following is considered a "service or support shop" in a manufacturing organization?
a) Foundry shop
b) Machine shop
c) Tool room
d) Welding shop

- (Very Difficult) In the context of concurrent engineering, the design approach integrates design and manufacturing to optimize:
a) Only the manufacturing cost.
b) All aspects of the product's life cycle.
c) Only the product's aesthetic features.
d) The separation of design and production phases.

Chapter 6: Metal Machining: Processes and Machine Tools

- (Easy) Which operation on a lathe is used to reduce the diameter of a cylindrical job?
a) Facing
b) Turning
c) Boring
d) Knurling

- (Easy) The cutting tool used on a drilling machine is called:
a) Reamer
b) Twist drill
c) Milling cutter
d) Single-point tool

- (Normal) What is the purpose of providing rake angle on a cutting tool?
a) To prevent rubbing of the flank on the workpiece.
b) To allow chips to flow away and reduce cutting forces.
c) To increase the strength of the cutting edge.
d) To improve the surface finish by burnishing.

- (Normal) In milling, the process where the cutter rotates against the direction of workpiece feed is called:
a) Down milling
b) Climb milling
c) Up milling
d) Face milling

- (Difficult) The phenomenon of "built-up edge" formation on a cutting tool is promoted by:
a) Low cutting speed and high friction.
b) High cutting speed and large rake angle.
c) Excessive use of cutting fluid.
d) Machining brittle materials like cast iron.

- (Difficult) In the Merchant circle diagram for orthogonal cutting, the shear force (Fs) is related to cutting force (Ft) and feed force (Ff) by which equation?
a) Fs = Ft cos φ + Ff sin φ
b) Fs = Ft sin φ - Ff cos φ
c) Fs = Ft cos φ - Ff sin φ
d) Fs = Ft sin φ + Ff cos φ

- (Very Difficult) Which type of milling cutter is best suited for cutting deep slots and has teeth with alternating helix to prevent chatter?
a) Plain milling cutter
b) Staggered tooth side milling cutter
c) Angle milling cutter
d) Fly cutter

- (Very Difficult) According to Taylor's tool life equation (VT^n = C), the exponent 'n' for a cemented carbide tool is typically in the range of:
a) 0.1 to 0.15
b) 0.2 to 0.5
c) 0.6 to 1.0
d) 1.0 to 1.5

- (Normal) The machine tool primarily used for producing flat surfaces on large and heavy workpieces is a:
a) Shaper
b) Planer
c) Slotter
d) Broaching machine

Chapter 13: Non-conventional Machining Methods

- (Easy) Which non-conventional machining process uses a controlled electrical spark to erode material from a conductive workpiece?
a) Electric Discharge Machining (EDM)
b) Ultrasonic Machining (USM)
c) Laser Beam Machining (LBM)
d) Abrasive Jet Machining (AJM)

- (Easy) In Electrochemical Machining (ECM), the workpiece acts as which electrode?
a) Cathode
b) Anode
c) Neither, it is not part of the circuit
d) Both, depending on polarity

- (Normal) The term LASER stands for:
a) Light Amplification by Stimulated Emission of Radiation
b) Light Amplification by Synchronized Emission of Radiation
c) Light Absorption by Stimulated Emission of Radiation
d) Light Amplification by Spontaneous Emission of Radiation

- (Normal) Ultrasonic Machining (USM) is particularly suitable for machining:
a) Very ductile metals like aluminium.
b) Hard and brittle materials like ceramics and glass.
c) Large ferrous workpieces.
d) Plastics and polymers.

- (Difficult) In Electron Beam Machining (EBM), why is a vacuum chamber required?
a) To cool the electron beam gun.
b) To prevent electron beam scattering by air molecules.
c) To increase the electrical conductivity of the workpiece.
d) To enhance the chemical etching rate.

- (Very Difficult) Which of the following non-conventional processes uses a high-velocity jet of ionized gas (plasma) to cut metal?
a) Ion Beam Machining (IBM)
b) Plasma Arc Machining (PAM)
c) Chemical Machining (CHM)
d) Electrochemical Grinding (ECG)

Chapter 15: Manufacturing Gears and Threads

- (Easy) Which gear has teeth cut parallel to the axis of rotation?
a) Helical gear
b) Spur gear
c) Bevel gear
d) Worm gear

- (Easy) The module (m) of a gear is defined as the ratio of:
a) Number of teeth to pitch circle diameter
b) Pitch circle diameter to number of teeth
c) Addendum to dedendum
d) Circular pitch to π

- (Normal) The most popular and fastest gear cutting process for mass production is:
a) Gear forming on milling machine
b) Gear hobbing
c) Gear shaping with a pinion cutter
d) Broaching

- (Normal) Which thread form is stronger and easier to manufacture than square threads and is commonly used for lead screws?
a) Acme threads
b) Buttress threads
c) Knuckle threads
d) Whitworth threads

- (Difficult) The process of finishing gear teeth by running the gear in mesh with a hardened shaving cutter that has serrations on its face is called:
a) Gear lapping
b) Gear shaving
c) Gear burnishing
d) Gear honing

- (Very Difficult) For cutting a helical gear on a milling machine using a form cutter, the cutter selection is based on the "equivalent number of teeth" (Nc) given by:
a) Nc = N / sin(θ)
b) Nc = N / cos³(θ)
c) Nc = N * cos(θ)
d) Nc = N * tan(θ)

Chapter 16: Jigs and Fixtures

- (Easy) The primary difference between a jig and a fixture is that a jig:
a) Is heavier in construction.
b) Guides the cutting tool.
c) Is used only for turning operations.
d) Does not hold the workpiece.

- (Easy) The "3-2-1" method of location is used to restrict how many degrees of freedom?
a) Six
b) Nine
c) Twelve
d) Three

- (Normal) The hardened steel bushes used in a drill jig to locate and guide the drill are called:
a) Drill bushes
b) Locating pins
c) Clamping pads
d) Jig feet

- (Normal) Which type of jig is used when holes need to be drilled on two or more sides of a workpiece, and the jig is tumbled?
a) Leaf jig
b) Plate jig
c) Template jig
d) Box jig

- (Difficult) A "foolproof" design in a jig or fixture ensures that:
a) The workpiece can only be loaded in one correct position.
b) The operator is protected from all moving parts.
c) The fixture is immune to wear and tear.
d) No cutting fluid can enter the clamping area.

- (Very Difficult) Which of the following is a principle of location that states locating points should be placed as far apart as possible to minimize the effect of wear?
a) Principle of extreme positions
b) Principle of mutually perpendicular planes
c) Principle of least points
d) Principle of three-point location

Chapter 17: Metal Joining Processes: Adhesive Bonding and Mechanical Fastening

- (Easy) Which of the following is a permanent type of mechanical fastener?
a) Bolt and nut
b) Screw
c) Rivet
d) Pin

- (Easy) The ability of an adhesive to cause one coated surface to adhere to another upon immediate contact is called:
a) Wet strength
b) Tack
c) Pot life
d) Case stability

- (Normal) Which type of adhesive is known for its rapid cure (within seconds) and is used in surgery?
a) Epoxies
b) Cyanoacrylates
c) Phenolics
d) Silicones

- (Normal) The process of joining two pieces of sheet metal by folding the edges together without using a separate fastener is called:
a) Crimping
b) Seaming
c) Stitching
d) Riveting

- (Difficult) According to the text, which adhesive system is known for having high strength and high temperature properties up to 200°C and is used for automotive brake lining?
a) Polyurethane-based
b) Epoxy-based
c) Acrylic-based
d) Cyanoacrylate-based

- (Difficult) Anaerobic adhesives harden satisfactorily only in:
a) The absence of atmospheric oxygen.
b) The presence of high humidity.
c) Intense ultraviolet light.
d) Elevated temperatures above 300°C.

- (Very Difficult) Which type of mechanical fastener is designed to be installed from one side of the assembly when the other side is inaccessible?
a) Cotter pin
b) Blind rivet
c) Self-tapping screw
d) Retaining ring

- (Very Difficult) The limitation of adhesive bonding regarding service life and reliability is primarily due to:
a) High cost of the adhesive material.
b) Deterioration of properties by environmental conditions over time.
c) Inability to join dissimilar materials.
d) Requirement of elaborate jigs and fixtures.

| Question | Correct Answer | Chapter | Book Page |
| --- | --- | --- | --- |
| 1 | b | 1 | 4 |
| 2 | c | 1 | 4 |
| 3 | a | 1 | 6-7 |
| 4 | c | 1 | 18 |
| 5 | b | 1 | 7 |
| 6 | b | 6 | 351 |
| 7 | b | 6 | 390 |
| 8 | b | 6 | 282 |
| 9 | c | 6 | 433 |
| 10 | a | 6 | 290-291 |
| 11 | c | 6 | 299 |
| 12 | b | 6 | 437 |
| 13 | b | 6 | 315 |
| 14 | b | 6 | 417-418 |
| 15 | a | 13 | 775 |
| 16 | b | 13 | 788 |
| 17 | a | 13 | 780 |
| 18 | b | 13 | 784 |
| 19 | b | 13 | 777 |
| 20 | b | 13 | 779 |
| 21 | b | 15 | 813 |
| 22 | b | 15 | 817 |
| 23 | b | 15 | 824 |
| 24 | a | 15 | 830 |
| 25 | b | 15 | 826 |
| 26 | b | 15 | 821 |
| 27 | b | 16 | 840 |
| 28 | b | 16 | 850 |
| 29 | a | 16 | 861 |
| 30 | d | 16 | 865 |
| 31 | a | 16 | 849 |
| 32 | a | 16 | 853 |
| 33 | c | 17 | 884 |
| 34 | b | 17 | 878 |
| 35 | b | 17 | 880 |
| 36 | b | 17 | 886 |
| 37 | b | 17 | 879 |
| 38 | a | 17 | 880 |
| 39 | b | 17 | 885 |
| 40 | b | 17 | 883 |