MCQs

Chapter 1: Manufacturing Principles and Processes

- (Easy) What is the process of converting raw materials into a usable form of product called?
a) Production method
b) Technology
c) Manufacturing
d) Productivity

- (Normal) Which production method concentrates on producing one or several articles like prototypes and is usually more expensive?
a) Unit production
b) Batch production
c) Mass production
d) Continuous production

- (Difficult) According to the text, which of the following is a key attribute considered in the decision-making exercise for a manufacturing system?
a) Automation
b) Flexibility
c) Standardization
d) Mechanization

- (Very Difficult) The concept of "Just-in-Time" (JIT) production primarily aims to ensure that:
a) High-quality products are made in time and at low cost.
b) The supply of raw material is done in large batches.
c) Quality is inspected into the product after fabrication.
d) Manufacturing and design activities take place sequentially.

Chapter 5: Foundry Processes: Molding and Casting

- (Easy) What is the purpose of a riser in a sand mold?
a) To act as a passage for molten metal to enter the mold cavity.
b) To provide a path for gases to escape.
c) To compensate for shrinkage of the casting during solidification.
d) To create a hollow internal cavity in the casting.

- (Easy) Which of the following is a hand tool used by a molder to compact sand in a molding box?
a) Swab
b) Rammer
c) Gate knife
d) Trowel

- (Normal) The process of introducing a "modifier" into molten metal to improve the size and shape of grains is known as:
a) Inoculation
b) Grain refining
c) Alloying
d) Annealing

- (Normal) Castings with extremely hard and wear-resistant surfaces are produced by which method?
a) Investment casting
b) Die casting
c) Chill casting
d) Centrifugal casting

- (Difficult) What is the primary function of "chaplets" in a mold?
a) To form the gating system.
b) To create a vent for gases.
c) To support and anchor cores in the mold cavity.
d) To provide a protective coating on the mold face.

- (Very Difficult) According to the text, a non-pressurized gating system typically has a gating ratio like 1:2:2 to achieve which of the following?
a) A back pressure to keep the system full of metal.
b) Reduced metal velocity and minimized turbulence.
c) Rapid filling of the mold with sufficient pressure.
d) A smaller volume of metal left in the gating system.

Chapter 7: Electric and Gas Welding Processes

- (Easy) Which arc welding process uses a continuously fed consumable electrode and a shielding gas like argon or helium?
a) Shielded Metal-Arc Welding (SMAW)
b) Gas Metal-Arc Welding (GMAW or MIG)
c) Submerged Arc Welding (SAW)
d) Gas Tungsten Arc Welding (GTAW or TIG)

- (Easy) What is the function of the flux coating on an electrode in shielded metal-arc welding?
a) To increase the melting rate of the electrode.
b) To provide a protective gaseous shield and form a slag.
c) To act as a deoxidizer for the base metal only.
d) To increase the electrical conductivity of the arc.

- (Normal) The phenomenon of a strong magnetic field deflecting the arc during DC welding is called:
a) Polarity
b) Arc blow
c) Arc stability
d) Backfire

- (Normal) In resistance spot welding, the "squeeze time" is the period for:
a) The flow of welding current.
b) Applying full electrode force before the current is turned on.
c) Maintaining pressure after the current is turned off.
d) The interval between the end of one cycle and the start of the next.

- (Difficult) "Weld decay" in stainless steel is a phenomenon primarily associated with:
a) Cracking in the weld metal due to hydrogen embrittlement.
b) The precipitation of chromium carbides at grain boundaries in the HAZ.
c) The formation of a hard and brittle martensitic structure in the weld.
d) The loss of alloying elements due to high-temperature vaporization.

- (Very Difficult) The exothermic chemical reaction in thermit welding for steel typically involves aluminium and:
a) Magnetic iron oxide (Fe₃O₄)
b) Silicon dioxide (SiO₂)
c) Calcium carbide (CaC₂)
d) Iron sulphide (FeS)

- (Very Difficult) Which of the following metals is forge-welded most successfully?
a) High carbon steel
b) Wrought iron and low carbon steels
c) Cast iron
d) Stainless steel

Chapter 8: Soldering and Brazing

- (Easy) The primary difference between soldering and brazing is based on the:
a) Type of flux used.
b) Melting point of the filler metal.
c) Strength of the final joint.
d) Method of applying heat.

- (Normal) In soldering and brazing, the distribution of the molten filler metal between the joint surfaces is achieved by:
a) External pressure applied by the operator.
b) Capillary action.
c) Mechanical interlocking of the joint design.
d) The weight of the filler metal itself.

- (Difficult) For brazing which of the following metals would a flux composed of fluorides and chlorides be most appropriate?
a) Carbon steel
b) Aluminium and its alloys
c) Copper
d) Cast iron

Chapter 9: Metal Forming: Hot- and Cold-working and Press-working

- (Easy) Hot-working of metals is carried out at a temperature:
a) Below the recrystallization temperature.
b) Above the recrystallization temperature.
c) Just above the melting point.
d) At room temperature.

- (Easy) Which cold forming process is used to produce threads on bolts and screws without any material wastage?
a) Cold heading
b) Thread rolling
c) Swaging
d) Coining

- (Normal) The process of forcing a billet through a die to produce a long, constant cross-section product is known as:
a) Rolling
b) Drawing
c) Extrusion
d) Forging

- (Normal) Which type of rolling mill uses small diameter work rolls supported by larger back-up rolls to reduce deflection and is common for cold rolling?
a) Two-high reversing mill
b) Three-high mill
c) Four-high mill
d) Planetary mill

- (Difficult) The term "alligatoring" refers to a defect found in:
a) Rolled plates and sheets.
b) Extruded products.
c) Forgings.
d) Welded joints.

- (Very Difficult) In wire drawing, the distinction between a rod and a wire is somewhat arbitrary, but the term "wire" is generally used for products with a diameter:
a) Greater than 5 mm.
b) Less than 5 mm.
c) Greater than 10 mm.
d) Drawn on a draw bench instead of a bull block.

Chapter 10: Metal Forging: Smithying and Power Forging

- (Easy) Which tool is used by a blacksmith as a supporting surface for hammering hot metal?
a) Swage block
b) Anvil
c) Bick iron
d) Hardy

- (Normal) The forging operation that increases the cross-sectional area of a workpiece at the expense of its length is called:
a) Drawing down
b) Fullering
c) Upsetting
d) Swaging

- (Difficult) In open die forging, the characteristic barreling effect on a workpiece is primarily caused by:
a) The impact speed of the hammer.
b) Frictional forces at the die-workpiece interface.
c) The initial temperature of the workpiece.
d) The rate of cooling after forging.

- (Very Difficult) A "cold shut" or "fold" in a forging is a defect that occurs due to:
a) Working the metal at too low a temperature.
b) Sharp corners in the die cavity causing metal to fold against itself.
c) The use of an oversize billet for the die.
d) Cracking at the flash line during trimming.

Chapter 12: Plastics: Manufacturing and Applications

- (Easy) Which type of plastics can be softened and reshaped repeatedly by heating?
a) Thermoplastics
b) Thermosetting plastics
c) Elastomers
d) Reinforced plastics

- (Easy) The process used to make long, continuous shapes like pipes, rods, and window frames from thermoplastics is called:
a) Injection molding
b) Blow molding
c) Extrusion
d) Calendering

- (Normal) The chemical reaction by which single monomer molecules are linked to form long-chain polymer molecules is known as:
a) Polymerization
b) Cross-linking
c) Curing
d) Plastification

- (Normal) Which of the following is an example of a thermosetting plastic?
a) Polyethylene
b) Epoxy (Araldite)
c) PVC
d) Nylon

- (Difficult) Which property makes thermosetting plastics generally stronger and more heat-resistant than thermoplastics?
a) Linear polymer chains with weak secondary bonds.
b) A cross-linked, three-dimensional network structure.
c) The addition of plasticizers for flexibility.
d) Their ability to be melted and re-molded.

- (Difficult) Which reinforcement fibre is noted for having the highest strength-to-weight ratio of all fibres, but its properties are degraded by moisture absorption?
a) Glass fibres
b) Graphite fibres
c) Aramids
d) Boron fibres

- (Very Difficult) Sheet Molding Compound (SMC) is a preformed raw material for reinforced plastics made by:
a) Chopping continuous fibres and depositing them on a layer of resin paste.
b) Injecting molten plastic into a closed mold.
c) Extruding a parison and inflating it in a mold.
d) Calendering a heated donghy paste into a sheet.

- (Very Difficult) Which plastic is known for its outstanding optical properties, clarity, and is used for aircraft canopies and lenses?
a) Polycarbonate
b) Acrylics
c) Fluorocarbons (Teflon)
d) ABS

- (Difficult) The presence of a V-shaped stinger or impingement in a die is a characteristic feature of which fine blanking process?
a) Progressive die blanking
b) Compound die blanking
c) Fine blanking
d) Transfer die blanking

- (Normal) The operation of placing configurations on a sheet metal surface by drawing the material into position with little effect on thickness is called:
a) Flanging
b) Beading
c) Embossing
d) Hemming

Question	Correct Answer	Chapter	Book Page

1	c	1	2

2	a	1	4

3	b	1	6

4	a	1	19

5	c	5	169

6	b	5	174

7	c	1	18

8	c	5	218

9	c	5	193

10	b	5	244

11	b	7	536

12	b	7	524

13	b	7	512

14	b	7	543

15	b	7	603

16	a	7	551

17	b	7	560

18	b	8	610

19	b	8	610

20	b	8	619

21	b	9	627

22	b	9	655

23	c	9	644

24	c	9	638

25	a	9	642

26	b	9	659

27	b	10	698

28	c	10	704

29	b	10	711

30	b	10	727

31	a	12	750

32	c	12	763

33	a	12	747

34	b	12	758

35	b	12	749

36	c	12	753

37	a	12	766

38	b	12	755

39	c	9	679

40	c	9	689